Robotics

Introduction

This project involves simulating a robot that separates different-coloured dumbbells from other items and places the dumbbells in the bin.

Environment

The environment is enclosed by walls and features bins, collectables, obstacles and the robot itself. Dumbbells are randomly spawned in weights ranging from 5 to 18 kg for each colour. This spawning occurs every 15 to 60 seconds until the total weight of spawned dumbbells exceeds 100 kg. There are four bins, one for each colour, each equipped with a central opening that allows the robot to deposit the dumbbells. The bins also provide updates when they’re filled with dumbbells, including the total weight they contain. (can be simplified if dumbbells are spawned in world file with their details on launch)

Vision:

# vision details go here, like how the robot was trained and how the camera is used for “Mark”

Navigation:

The sortabot navigates using its LIDAR sensor to detect obstacles and move in discrete steps. Whenever the robot decides to roam, it chooses to turn at an angle between 45 degrees clockwise and anticlockwise from where it is currently facing. Should it encounter an obstacle it will try turning at an angle between 70 and 80 degrees on one side, if it still cannot move it'll try 90 degrees and if that fails it will face the direction it came from.

To move efficiently the sortabot uses vision to mark locations of dumbbells and bins as well as label them, this is the “Mark” action in the reinforcement learning aspect. It then has a “GoTo” method which uses A* to map the shortest route to a previously marked dumbbell or bin based on its need. The method uses LIDAR data collected during traversal to figure out the layout of the area. If traversal is blocked by a previously unseen obstacle the sortabot will reorient as it does in its regular roaming and try again until it finds a viable way to its target destination. (can be simplified if the robot already knows bin locations and the dumbbells are spawned at the start)

Weight Checking:

The sortabot can lift only up to 12kg with a single hand, as such in the event that the target object wieght exceeds this it must either drop the weight and come back for it later or lift it using both of its hands.

Reinforcement Learning:

There are a number of discrete actions the sortabot may opt to do: “Roam” in which it performs regular navigation; “Mark” by which sortabot moves its camera and records any dumbbell object or bin's current position on the occupancy grid, for the sake of adaptability this will always overwrite any previous markings; “GoTo” in which the sortabot plots and traverses the shortest path to a dumbbell/bin via A*; “pickUp” in which the sortabot chooses a hand (or both in the instance of heavy dumbbells) to pick up a dumbbell object; “putDown” in which the sortabot releases a dumbbell object from its hand or hands. (Can be simplified by not including RL and just having the robot go to and pick up objects the moment they see one, and rush to the appropriate bin the moment they have an object in hand)